# Daily Excavation Safety Checklist

| Company |  | Date |  |
| --- | --- | --- | --- |
| Project Name |  | Approx. Temp. |  |
| Project Location |  | Approx. Wind Dir. |  |
| Job Number |  | Safety Rep |  |
| Excavation Depth & Width |  | Soil Classification |  |
| Protective System Used |  |  |  |
| Activities In Excavation |  |  |  |
| Competent Person |  |  |  |

Excavation > 4 feet deep? ___Yes ___No If YES, fill out a Confined Space Permit PRIOR to ANY person entering the excavation.

NOTE: Trenches over 4 feet in depth are considered excavations. Any items marked NO on this form MUST be remediated prior to any employees entering the excavation.

| YES | NO | N/A | ### DESCRIPTION |
| --- | --- | --- | --- |
| GENERAL |  |  |  |
|  |  |  | Employees protected from cave-ins & loose rock/soil that could roll into the excavation |
|  |  |  | Spoils, materials & equipment set back at least 2 feet from the edge of the excavation. |
|  |  |  | Engineering designs for sheeting &/or manufacturer's data on trench box capabilities on site |
|  |  |  | Adequate signs posted and barricades provided |
|  |  |  | Training (toolbox meeting) conducted w/ employees prior to entering excavation |

| YES | NO |  | N/A | ### DESCRIPTION |
| --- | --- | --- | --- | --- |
| UTILITIES |  |  |  |  |
|  |  |  |  | Utility company contacted & given 24 hours notice &/or utilities already located & marked |
|  |  |  |  | Overhead lines located, noted and reviewed with the operator |
|  |  |  |  | Utility locations reviewed with the operator, & precautions taken to ensure contact does not occur |
|  |  |  |  | Utilities crossing the excavation supported, and protected from falling materials |
|  |  |  |  | Underground installations protected, supported or removed when excavation is open |
| WET CONDITIONS |  |  |  |  |
|  |  |  |  | Precautions taken to protect employees from water accumulation (continuous dewatering) |
|  |  |  |  | Surface water or runoff diverted /controlled to prevent accumulation in the excavation |
|  |  |  |  | Inspection made after every rainstorm or other hazard increasing occurrence |
| HAZARDOUS ATMOSPHERES |  |  |  |  |
|  |  |  |  | Air in the excavation tested for oxygen deficiency, combustibles, other contaminants |
|  |  |  |  | Ventilation used in atmospheres that are oxygen rich/deficient &/or contains hazardous substances |
|  |  |  |  | Ventilation provided to keep LEL below 10 % |
|  |  |  |  | Emergency equipment available where hazardous atmospheres could or do exist |
|  |  |  |  | Safety harness and lifeline used |
|  |  |  |  | Supplied air necessary (if yes, contact safety department) |

| YES | NO | N/A | ### DESCRIPTION |
| --- | --- | --- | --- |
| ENTRY & EXIT |  |  |  |
|  |  |  | Exit (i.e. ladder, sloped wall) no further than 25 feet from ANY employee |
|  |  |  | Ladders secured and extend 3 feet above the edge of the trench |
|  |  |  | Wood ramps constructed of uniform material thickness, cleated together @ the bottom |
|  |  |  | Employees protected from cave-ins when entering or exiting the excavation |

## Keep 1 copy of each daily excavation checklist on site for the project duration, and forward the original to the safety director