Instructions:  A competent person qualified to recognize fall hazards and who knows the fall protection rules must write a plan for this worksite if a fall hazard(s) of 6 feet or more will exist while employees are on site.   Fill in the blanks and check the appropriate boxes.   A copy of this plan must be on the worksite while employees are on site.

Worksite information

Company Name: ____________________________________  Date Prepared: ____/___/____

Worksite Address: ________________________________________ City: ________________

Competent Person to Supervise the Plan: __________________________________________

Where will this plan will be posted? _______________________________________________

Fall protection methods                 Identified Fall hazards above 6 feet

Check all that apply

|  Standard Guardrails: Are to be made from 2 X 4’s with posts spaced no more than 8’ apart. The top rail must be 39" to 45" above the work surface with a midrail at the half-way point and a toe board if workers will walk or work below. The rail must withstand 200 pounds of pressure on the top rail in any direction. Note: A guardrail does not protect a person standing on a ladder, stilts, a box or other surface above the work surface. Additional Instructions: |  Roof > 4/12 Pitch  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Window Opening  Open Sided Floor  Deck / balcony  Leading Edge  Scaffold Work  Mobile Lift Work  Ladder Work  Excavation Edge  Grade Drop-Off  Other:________ |
| --- | --- |
|  Fall Arrest Harness: Anchor points must be capable of withstanding 5000 pound shock unless a deceleration device in use limits free fall to 2 feet in which case a 3000 pound anchor point may be used. Anchors points are: _________________________________________ Configuration and placement sketch attached:  Yes Manufacturer’s installation instructions attached?  Yes Free fall may not exceed 6'. A lower level may not be contacted during a fall. Lifelines must be placed or protected to prevent abrasion damage. Snap hooks may not be connected to each other, or to loops in webbing. Harness manufacturer: ______________________________________ Manufacturer’s use and care instructions attached?  Yes Additional Instructions: |  Roof > 4/12 Pitch  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Open Beam  Truss/Top Plate  Window Opening  Open Sided Floor  Deck / Balcony  Leading Edge  Scaffold Work  Mobile Lift Work  Ladder Work  Excavation Edge  Grade Drop-Off  Other:________ |

Fall protection methods                 Identified Fall hazards above 6 feet

Check all that apply

|  Fall Restraint Harness: Anchor points must be able to withstand 4 times the body weight with tools of the heaviest person who will use the system. The system must always be rigged to prevent a free fall from the work surface. Several alternate anchor points may be necessary to prevent free fall from any location. If a horizontal catenary line is used, it will be designed by a competent person who recognizes that a horizontal line can multiply the force applied to the anchor. Anchors points are: _________________________________________ Configuration and placement sketch attached:  Yes Manufacturer’s installation instructions attached?  Yes Harness manufacturer: ______________________________________ Manufacturer’s use and care instructions attached?  Yes Additional Instructions: |  Roof > 4/12 Pitch  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Window Opening  Open Sided Floor  Deck / Balcony  Leading Edge  Scaffold Work  Mobile Lift Work  Excavation Edge  Grade Drop-Off  Other:________ |
| --- | --- |
|  Warning Line System: A line made up of  rope  wire or  chain between 36" - 42" above the surface flagged at 6 foot intervals will be attached to stanchions such that pulling on one section of chain will not take up slack in the other sections. The stanchions must be able to withstand a 16 pound force applied horizontally at 30" high. The warning line will be erected at least 6 feet back from the fall hazard. It will be set up so that a person entering will be directed to the work zone and prevented from entering unprotected areas. Configuration and placement sketch attached:  Yes Additional Instructions: |  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Window Opening  Open Sided Floor  Deck / Balcony  Leading Edge  Other:________ |

Fall protection methods                 Identified Fall hazards above 6 feet

Check all that apply

|  Hole/opening cover: It must be able to support 2 X the weight of employees and equipment. It will be secured to prevent accidental displacement. We will use  hinges  cleats nailed to the underside of the cover  Nail or screw it in place. It will be marked "Cover" on top. Our covers will be made from: __________________________________ Additional Instructions: |  Skylight Opening  Roof Opening  Floor Opening  Other:________ |
| --- | --- |
|  Catch Platform: Must be installed within 10' vertical of the work area. Must be as wide as the fall distance (minimum 45”). Must have guardrails on all open sides. Additional Instructions: |  Roof > 4/12 Pitch  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Open Beam  Truss/Top Plate  Window Opening  Open Sided Floor  Deck / Balcony  Leading Edge  Other:________ |
|  Safety Net: Must be installed within 30 feet vertical of the work surface and must extend out from the outermost projection of the work surface: Up to 5' Fall = 8 Feet Actual vertical fall distance: ____________ 5' to 10' Fall = 10 Feet > 10' Fall = 13 Feet Actual net width: _______________ A person falling into the net can't contact any object below the net. The system must be tested or certified to withstand a 400 pound object dropped from the highest work surface. Mesh at any point must not exceed 36 square inches with the largest opening being 6" side to side. Inspect weekly for mildew, wear or damage and remove any objects in net as soon as possible. Anchors points are: _________________________________________ Configuration and placement sketch attached:  Yes Manufacturer’s installation instructions attached?  Yes Additional Instructions: |  Roof > 4/12 Pitch  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Open Beam  Truss/Top Plate  Window Opening  Open Sided Floor  Deck / Balcony  Leading Edge  Scaffold Work  Mobile Lift Work  Ladder Work  Excavation Edge  Grade Drop-Off  Other:________ |

Fall protection methods                 Identified Fall hazards above 6 feet

Check all that apply

|  Positioning Belt: A full body harness will be used with two lanyards attached to “D” rings on each side of the employee’s waist. The employee will attach both lanyards to an anchor point and be rigged to not be able to fall more than 2 feet . The anchor must be able to sustain 2 times the intended load or 3000 lbs whichever is greater. Snap hooks must not be connected to each other or to loops in webbing. Anchors points are: _________________________________________ Configuration and placement sketch attached:  Yes Manufacturer’s installation instructions attached?  Yes Additional Instructions: |  Roof > 4/12 Pitch  Open Beam  Truss/Top Plate  Window Opening  Ladder Work  Work from Wall  Other:_______ |
| --- | --- |
|  Other Fall Protection System: Provide a description of how the system is to be assembled, disassembled, operated, inspected and maintained including specifications for materials to be used in its construction: |  Roof > 4/12 Pitch  Roof =< 4/12 Pitch  Skylight Opening  Roof Opening  Floor Opening  Open Beam  Truss/Top Plate  Window Opening  Open Sided Floor  Deck / Balcony  Leading Edge  Scaffold Work  Mobile Lift Work  Ladder Work  Excavation Edge  Grade Drop-Off  Other:________ |
| Note: Fall protection systems will be assembled and maintained according to manufacturer's instructions when using a manufactured system. A copy of those instructions are available on site for reference. |  |

Protection Against Overhead Hazards

When employees are working at elevation, we will provide the following protection(s) for employees working below:

 Require persons to wear hard hats		 Screens on Guardrails

 Overhead Hazard Signs				 Toe Boards on Guardrails

 Debris Nets						 Barricade to Control Access

 Other _____________________________________________________________________

Handling Storing and Securing Tools and Materials

All  personal fall protection equipment will be kept  at ______________________  to prevent damage from weather and other sources.  Damaged fall protection equipment must be reported to the supervisor immediately and must be repaired or replaced before employees may enter the area containing the fall hazard.

Tools used at elevations will be secured as follows:

 Tool belts will be used to carry hand tools.  Tools will be returned to the belt after use.

 Tools too large for the tool belt will be raised by rope and pulley.

 Large tools which may fall to lower levels will be secured to rails and other fixed objects where necessary.

 Other: ___________________________________________________________________

Building materials will be delivered to upper levels by the following methods:

___________________________________________________________________________

Storage at elevation will not exceed 1 unit high.

Other Instructions: ____________________________________________________________

Emergencies and Injuries

First Aid Trained Employee(s) on Site:

Name _______________________________ Name __________________________________

First Aid Kit Location(s): _________________________________________________

Nearest Medical Facility: _________________________________________________

Emergency Services Phone Numbers:

Medical ________________ Fire ____________________  Police ________________

Location of Nearest Telephone:  ___________________________________________

If a crew member is injured at elevation, the supervisor will evaluate the employee's condition and may voluntarily administer first aid.  Emergency services will be called as needed.  If an injured employee can't return to ground level  the employee will be brought down to a lower level by  emergency services.

The following equipment is available on site to help lower the injured worker:

_____________________________________________________________________

Employee Training

We have explained this fall protection plan to each employee at the job site.  Employees who use personal fall protection equipment have received training in its proper use and care. By signing this document, the employees acknowledge that they understand the plan and have been trained in the use of the equipment.

| Name | Signature | Date |
| --- | --- | --- |

By signing below, the competent person certifies that the fall hazard analysis has been done,  the employees have been trained on the plan and that employees have received training in how to use and care for the personal fall protection equipment they were given:

| Name | Signature | Date |
| --- | --- | --- |